NATIONAL UNIVERSITY OF COMPUTER

AND EMERGING SCIENCES (FAST-NUCES)

Department of Electrical Engineering

PROJECT TECHNICAL REPORT

AC Motor Starter Circuit

| Course | EL2010 – Electromechanical Systems Lab |
| --- | --- |
| Session | Spring 2026 |
| Submitted To | Sir Amir Ali |
| Submission Date | April 30, 2026 |
| Institution | FAST-NUCES |
| Project Type | Open-Ended CEA Lab Project |

Group Members

Farhan  (23K-6036)    |    Abbas  (23K-6049)    |    Afshan Malik  (23K-6068)    |    Hasnain  (23K-6061)

# Abstract

This project presents the design and implementation of a Direct-On-Line (DOL) starter for a single-phase induction motor using a control circuit with a Finder 60.12 Relay. The system incorporates multiple protection mechanisms including an overload relay, emergency stop push button, and a miniature circuit breaker (MCB). The developed setup ensures safe motor operation, intuitive control, and reliable performance. This project demonstrates practical understanding of industrial motor control systems and their real-world significance in electrical engineering.

# 1. Objective

The primary objective of this project is to design, build, and test a Direct-On-Line (DOL) starter for a single-phase induction motor using a relay-based control circuit equipped with essential safety features. The project aims to:

- Demonstrate the working principle of a DOL motor starter in a practical hardware setup.

- Implement safety measures including overload protection, emergency stop, and short circuit protection.

- Provide hands-on understanding of contactor-based motor control circuits.

- Align the project with SDG Goal 9 (Industry, Innovation and Infrastructure) by promoting efficient and safe motor control.

# 2. Introduction

A Direct-On-Line (DOL) starter is the simplest and most widely used method of starting a single-phase induction motor. It connects the motor directly to the full supply voltage at the moment of starting, thereby providing high starting torque. However, this method produces a significant inrush current — typically 5 to 8 times the rated current — which necessitates robust protective devices to prevent equipment damage.

In this project, a Finder 60.12 Relay (220V AC, 8-pin) is used alongside a contactor and an overload relay to ensure safe and efficient operation. The control circuit is designed to allow latching (self-holding) operation, meaning once started, the motor continues to run until a stop command is issued, making it ideal for industrial and commercial applications such as pumps, compressors, and conveyors.

# 3. Components Used

The following hardware components were used to construct the motor starter circuit:

| # | Component | Specification | Quantity |
| --- | --- | --- | --- |
| 1 | Miniature Circuit Breaker (MCB) | 1-Pole, 6A | 1 |
| 2 | Contactor (KM) | 220V AC Coil, Schneider LC1 | 1 |
| 3 | Overload Relay (OLR) | Adjustable, Thermal Type | 1 |
| 4 | Finder 60.12 Relay | 220V AC, 8-Pin, 2-CO | 1 |
| 5 | Relay Base/Socket | 8-Pin DIN Rail Mount | 1 |
| 6 | Stop Push Button | Normally Closed (NC), Red | 1 |
| 7 | Emergency Stop Button | Normally Closed (NC), Mushroom Head | 1 |
| 8 | Start Push Button | Normally Open (NO), Green | 1 |
| 9 | Capacitor Load Box | 4.7 µF – 1000 µF Selectable | 1 |
| 10 | Single-Phase Induction Motor | 120W, 220V, 1400 RPM | 1 |

# 4. Circuit Description

4.1 Power Circuit (220V AC)

The power circuit delivers electrical energy from the supply to the motor through switching and protection devices. The flow of power is as follows:

- 220V AC Phase Supply → MCB → Contactor Main Contacts → Overload Relay → Motor (Main Winding)

- The capacitor from the load box is connected between the Phase line and the starting winding (GREEN terminal) of the motor.

- Both common (BLACK) motor terminals are connected to the Neutral line.

4.2 Control Circuit (220V AC)

The control circuit governs the switching logic of the system. It operates at 220V AC and controls the contactor coil via the Finder relay. The control logic is:

- Phase → MCB → STOP (NC) → EMERGENCY STOP (NC) → OLR (NC) → START (NO) → Relay Coil (A1)

- Relay Neutral connection: Relay Coil (A2) → Neutral

- Relay NO Contact (1–4) provides a self-holding latch for the contactor coil output.

4.3 Relay (8-Pin) Wiring

- Pin 2 – Phase (L) Input

- Pin 8 – Coil A1 (connected after START button)

- Pin 7 – Coil A2 (Neutral)

- Pins 1 & 4 – Normally Open (NO) Contact: Output to Contactor Coil (A1)

# 5. Working Principle

The circuit operates in the following sequential steps:

| Step | Event | Description |
| --- | --- | --- |
| 01 | Press START | START button (NO) closes → Finder Relay coil energizes → Relay NO contact (1–4) closes. |
| 02 | Contactor Activates | Relay contact output energizes Contactor coil (A1) → Contactor main contacts close. |
| 03 | Motor Starts | Power flows through contactor to motor → Capacitor creates phase shift → Motor runs. |
| 04 | Press STOP | STOP (NC) opens → Relay de-energizes → Contactor opens → Motor stops. |
| 05 | Emergency Stop | Mushroom button pressed → Circuit breaks immediately → Motor halts safely. |
| 06 | Overload Trip | Excessive current heats OLR bi-metallic strip → OLR (NC) opens → Motor protected. |

# 6. Protection Features

- Provides instant protection against short-circuit faults by disconnecting the supply.MCB (Miniature Circuit Breaker):

- Monitors motor current and trips the circuit if it exceeds the rated value, preventing motor burnout.Overload Relay (OLR):

- A dedicated NC mushroom-head button allows immediate manual shutdown in hazardous situations.Emergency Stop Button:

- Electrically isolates the low-power control logic from the high-power contactor coil, improving safety.Finder Relay (Isolation):

# 7. Cost Analysis

| Component | Cost (PKR) |
| --- | --- |
| Contactor (Schneider LC1) | PKR 2,200 |
| Finder 60.12 Relay | PKR 400 |
| Relay Base / Socket | PKR 150 |
| Emergency Stop Button | PKR 150 |
| Start Push Button (NO) | PKR 150 |
| Stop Push Button (NC) | PKR 150 |
| Total Estimated Cost | PKR 3,200 |

# 8. SDG Alignment

This project aligns with the United Nations Sustainable Development Goal 9: Industry, Innovation and Infrastructure.

- Energy Efficiency: Motor starters reduce mechanical and electrical strain during startup, lowering energy consumption and extending motor lifespan.

- Industrial Safety: Overload protection and emergency stop mechanisms prevent accidents and protect both workers and machinery.

- Scalability: The DOL starter design can be adapted for larger industrial motors, contributing to safer and smarter industrial infrastructure.

- Skill Development: The hands-on implementation promotes engineering education and technical skill-building among students.

# 9. Advantages & Limitations

9.1 Advantages

- Extremely simple circuit design with readily available components.

- Low cost — suitable for small-scale and educational applications.

- Provides full starting torque, making it ideal for loads requiring high initial torque.

- Easy to install, troubleshoot, and maintain.

- Reliable operation with proper protection integration.

9.2 Limitations

- High inrush current (5–8× rated current) causes voltage dips in the supply line.

- Not suitable for large motors where reduced voltage starting is required.

- No speed control capability — motor operates at a single fixed speed.

- Mechanical stress on motor windings during direct-on-line starting.

# 10. Applications

DOL motor starters are widely deployed across industrial and commercial sectors:

- Water pumps and submersible pump systems

- Industrial conveyor belt drives

- Air compressors and refrigeration compressors

- Fans, blowers, and HVAC systems

- Textile and packaging machinery

# 11. Result

The DOL starter using the Finder 60.12 Relay was successfully constructed and tested. The motor started reliably upon pressing the START button, ran continuously due to the relay self-holding latch, and stopped immediately upon pressing STOP or EMERGENCY STOP. The overload relay was also verified to trip correctly under simulated overload conditions. All measurements were recorded using voltmeters and ammeters as per project requirements.

# 12. Conclusion

This project successfully demonstrates the design, construction, and operation of a Direct-On-Line (DOL) motor starter with comprehensive protection systems. The use of a Finder 60.12 Relay, contactor, and overload relay provides a robust and safe control solution for a single-phase induction motor. The circuit is cost-effective, reliable, and scalable, making it highly suitable for small and medium-scale industrial applications. The project reinforces core concepts of electromechanical systems and prepares students for real-world electrical engineering challenges.

# 13. Future Scope

- PLC-Based Automation: Replace relay logic with a Programmable Logic Controller for advanced sequencing and diagnostics.

- IoT Monitoring System: Integrate current/voltage sensors with a microcontroller and cloud platform for real-time remote monitoring.

- Soft Starter Integration: Implement a soft starter to gradually ramp up motor voltage, eliminating inrush current issues.

- Variable Frequency Drive (VFD): Add speed control capability for energy-efficient variable-speed operation.

- Remote Operation: Enable mobile-app or web-based remote start/stop functionality for smart facility management.

# References

[1]  Chapman, S. J., Electric Machinery Fundamentals, 5th ed., McGraw-Hill, 2012.

[2]  Schneider Electric, LC1D Series Contactor Datasheet, 2023.

[3]  Finder Group, Series 60 Relay Technical Manual, 2022.

[4]  IEC 60947-4-1: Low-voltage switchgear and controlgear – Contactors and motor-starters.